OptiMIPS

New Architecture Extensions Report

Forwarding Mode, Critical Path, Stall Profile, Register Pressure, and Loop-Aware Basic Blocks

Report purpose This report explains the five new features added to OptiMIPS in simple English. It explains what each feature does, why it matters in a MIPS pipeline optimizer, what logic was used in the code, and how the sample assembly program shows the feature in the GUI.

# 1. Project Context

OptiMIPS is a pipeline-aware MIPS code optimizer inside the MARS simulator. The original tool could parse assembled instructions, build dependencies, reorder safe instructions, and show CPI improvement. The new work improves the tool from a simple scheduler into a better teaching tool for computer architecture and compiler backend ideas.

The main idea is still the same: the tool must not change the meaning of the program. It can only move an instruction when dependency rules say the move is safe. The five new features add more realistic hardware modes, better explanation of stalls, deeper analysis of the dependency graph, register pressure visibility, and safer loop handling.

# 2. Quick Summary of the Five New Features

| Feature | What it shows | Why it matters | Main classes |
| --- | --- | --- | --- |
| Forwarding ON/OFF | CPI changes under two hardware modes. | Students can compare a forwarding pipeline with a stall-only pipeline. | Scheduler, OptiMIPS |
| Critical Path | Longest weighted dependency chain. | Shows the theoretical lower limit of scheduling. | DependencyGraph, CriticalPathResult |
| Stall Profile | Exact producer-consumer stall pairs. | Explains which hazards caused stalls and which were removed. | StallEvent, StallResult, Scheduler |
| Register Pressure | Live register count at each row. | Shows if optimization creates spill risk by keeping too many values live. | RegisterPressureAnalyzer, OptiMIPS |
| Loop-Aware Basic Blocks | Blocks split at labels, branches, jumps, and targets. | Lets loop bodies be optimized safely without crossing control-flow boundaries. | BasicBlock, BasicBlockSplitter |

# 3. Feature 1: Hardware Forwarding Mode

Forwarding is a hardware technique where a result is sent directly from a later pipeline stage to an earlier instruction that needs it. This avoids many stalls. In the GUI, the new toggle says Forwarding: ON or Forwarding: OFF.

When Forwarding is ON, normal R-type and I-type RAW hazards cost zero stall cycles because forwarding handles them. However, a load-use hazard still costs one stall when a load instruction is immediately followed by an instruction that uses the loaded register.

When Forwarding is OFF, the tool behaves like a simpler stall-only processor. Every RAW hazard costs latency minus one stall cycles. The latency values come from the Scheduler latency map.

| Mode | Rule | Student meaning |
| --- | --- | --- |
| Forwarding ON | Only immediate lw/lb/lh load-use hazards add 1 stall. | Modern pipeline behavior. |
| Forwarding OFF | Every RAW hazard adds latency - 1 stalls. | Simpler processor without bypassing. |

Code logic Scheduler.countStalls(instrs, forwardingEnabled) now returns a StallResult. The total stall count is used for CPI, while the event list is used by the Stall Profile tab.

# 4. Feature 2: Critical Path Analysis

The critical path is the longest dependency chain in the instruction graph. Even a perfect scheduler cannot run faster than this chain, because each instruction in the chain depends on earlier work.

OptiMIPS computes a weighted path length for each instruction. The weight is the instruction latency. Then it backtracks from the largest path length to find the actual critical path.

Topologically sort the dependency graph.

Process nodes from the end back toward the start.

For each instruction, compute latency plus the best successor path.

Start from the node with the maximum path length.

Follow the successor with the largest remaining path to build the critical path.

In the GUI, critical-path instructions get a red left border. The stats panel also shows the theoretical minimum CPI and the scheduling gap.

# 5. Feature 3: Stall Profile

The old CPI result only said how many stalls existed. The new Stall Profile explains exactly where the stalls came from. Each stall event stores the producer instruction, consumer instruction, hazard type, stall cycles, and source text.

The GUI compares stall events from the original sequence with stall events from the optimized sequence. If the same producer-consumer pair no longer causes a stall after optimization, the table marks it as YES in the Eliminated column.

# 6. Feature 4: Register Pressure Visualization

Instruction scheduling can reduce stalls, but it can also keep more registers live at the same time. If too many registers are live, a real compiler may need to spill values to memory. That is why register pressure is important.

The analyzer uses a simple liveness rule. At position i, a register is live if an earlier instruction wrote it and some instruction at or after i will read it. The pressure value is the number of live registers at that position.

| Pressure value | Bar color | Meaning |
| --- | --- | --- |
| 0 to 4 | Green | Low register pressure. |
| 5 to 8 | Orange | Moderate pressure. |
| 9 or more | Red | Possible spill risk. |

In the GUI, each code table has a thin pressure column on the right. The bar width is pressure divided by 16, multiplied by 40 pixels. A tooltip says how many live registers exist at that point.

# 7. Feature 5: Loop-Aware Basic Block Splitter

The original scheduler treated the assembled program almost like one large instruction list. It avoided moving instructions across branches, but it did not explicitly split the program into basic blocks. The new splitter fixes that.

A basic block is a straight-line sequence of instructions. Control enters at the first instruction and leaves at the end. The scheduler is allowed to reorder inside one block, but never across block boundaries.

The first instruction is a block leader.

Every label or branch target starts a new block.

Every instruction after a branch or jump starts a new block.

Each block gets its own DependencyGraph and Scheduler.

The optimized blocks are reassembled in the original block order.

Why this matters for loops Loop bodies run many times, so optimizing inside the loop body can improve the part of the program that matters most. At the same time, the block boundary prevents unsafe movement across loop branches.

# 8. Worked Example: optimips_feature_test.asm

The following program is designed to test all five new features. It has a loop, a branch target label, one load-use hazard, several RAW dependencies, independent filler instructions, and enough live registers to make pressure bars visible.

# OptiMIPS feature test program # Assemble this in MARS, then open Tools > OptiMIPS and click Optimize. # # What to look for: # - Forwarding ON/OFF changes CPI immediately. # - Load-use hazards appear in forwarding mode. # - RAW hazards grow in stall-only mode. # - Loop body is split into its own basic block. # - Critical path rows get a red left border. # - Register pressure bars grow in the loop body. # - Stall Profile shows which producer-consumer pairs were eliminated. .data array: .word 4, 8, 12, 16, 20, 24, 28, 32 result: .word 0 .text .globl main main: la $s0, array li $s1, 8 li $s2, 0 li $s3, 0 li $t7, 3 loop: lw $t0, 0($s0) # load-use hazard with next instruction add $t1, $t0, $s3 add $t2, $s2, $t7 # independent filler candidate sub $t3, $s1, $s2 # independent filler candidate and $t4, $t1, $t2 or $t5, $t4, $t3 xor $t6, $t5, $t0 add $s3, $s3, $t6 addi $s0, $s0, 4 addi $s2, $s2, 1 bne $s2, $s1, loop # loop branch creates block boundary after_loop: add $a0, $s3, $zero sw $a0, result li $v0, 10 syscall

## 8.1 What Happens in This Example

The loop label becomes a basic block leader.

The bne instruction ends the loop block.

The instruction after bne begins the after_loop block.

The lw followed by add creates a load-use hazard in Forwarding ON mode.

Independent instructions such as add $t2 and sub $t3 can move upward to fill waiting cycles.

The red critical-path border marks the longest dependency chain through the loop body.

The pressure bars become orange or red where many temporary registers are live.

## 8.2 Expected GUI Reading

| GUI area | How to read it |
| --- | --- |
| Forwarding toggle | Switch it ON and OFF. CPI changes immediately without rerunning the optimizer. |
| Original / Optimized code tables | Green moved rows show scheduling changes. Red left borders show critical-path rows. Faint horizontal lines show basic block boundaries. |
| Pressure bars | Bars on the right show how many registers are live at each row. Red means high pressure. |
| Critical path panel | Shows the best theoretical CPI and the remaining scheduling gap. |
| Stall Profile tab | Shows each stall pair. YES means the optimizer removed that exact stall pair. |
| Basic blocks panel | Shows how many blocks were found and each block's CPI before and after scheduling. |

# 9. How the Code Files Work Together

| File | Responsibility | Simple explanation |
| --- | --- | --- |
| MarsParser.java | Builds MipsInstruction objects. | Reads assembled MARS instructions and records registers, memory flags, source line, address, and branch target. |
| Scheduler.java | Schedules instructions and counts stalls. | Supports forwarding mode, stall-only mode, and StallResult event lists. |
| DependencyGraph.java | Protects program correctness. | Prevents unsafe movement and computes the critical path. |
| BasicBlockSplitter.java | Splits loops and branches into blocks. | Finds leaders, creates blocks, schedules each block independently, and reassembles output. |
| RegisterPressureAnalyzer.java | Counts live registers. | Shows whether optimization increases register pressure. |
| OptiMIPS.java | GUI and report surface. | Displays code, CPI, pressure bars, critical path, stall profile, and block summaries. |

# 10. Testing Steps

Open optimips_feature_test.asm in MARS.

Assemble the program with F3.

Open Tools > OptiMIPS.

Click Optimize.

Check that the forwarding toggle, critical path panel, stall profile tab, pressure bars, and basic block panel all update.

Toggle Forwarding OFF and confirm CPI and stall profile values refresh without rerunning Optimize.

# 11. Final Value of These Extensions

These five features make OptiMIPS more complete as a teaching tool. It no longer only shows that instruction scheduling can reduce CPI. It now explains why the CPI changes, which hardware mode is being modeled, which stalls were removed, which dependency chain limits the schedule, whether register pressure became risky, and how loop bodies are handled safely.

This is useful for a project report because it connects compiler backend logic with computer architecture concepts. The GUI becomes a learning surface, not just an optimizer output window.