# Feature 1: Hardware Forwarding Mode

Think of a processor pipeline like a factory assembly line.
Different instructions are being worked on at the same time in different stages.

Example pipeline stages:

- Fetch instruction

- Decode instruction

- Execute

- Memory access

- Write result back

1. What is a RAW Hazard?

RAW means:

Read After Write

It happens when one instruction needs a value that a previous instruction has not finished writing yet.

Example:

add r1, r2, r3     ; r1 = r2 + r3
sub r4, r1, r5     ; uses r1 immediately

The sub instruction needs r1, but the add instruction has not fully completed yet.

So the processor has a dependency problem.

2. What is a Stall?

A stall means:

👉 The pipeline pauses for some cycles and waits.

It is also called a bubble.

Example:

add r1, r2, r3
sub r4, r1, r5

Without any special hardware, sub must wait until add writes r1.

So processor inserts waiting cycles:

Cycle 1: add fetch
Cycle 2: add decode | sub fetch
Cycle 3: add execute | sub decode
Cycle 4: add memory  | STALL
Cycle 5: add write   | sub execute

That waiting is called a stall.

3. What is Forwarding?

Forwarding is a smart hardware trick.

Instead of waiting for the result to be fully written back to the register file, the processor directly sends the result to the next instruction.

So:

👉 Result is “forwarded” early.

Example with Forwarding ON

add r1, r2, r3
sub r4, r1, r5

The add computes result in Execute stage.

Processor directly forwards that result to sub.

So:

✅ No waiting
✅ No stall cycles

This is how modern processors work.

4. Load-Use Hazard (Special Case)

Loads are different because data comes from memory and arrives later.

Example:

lw r1, 0(r2)
add r3, r1, r4

add immediately needs r1.

But lw gets data from memory late.

Even with forwarding:

❌ Data is not ready immediately
✅ One stall is still needed

So:

| Situation | Stall |
| --- | --- |
| Normal add/sub dependency | 0 stall with forwarding |
| Immediate load-use dependency | 1 stall even with forwarding |

5. What is Stall-Only Processor?

This means:

❌ No forwarding hardware exists.

Processor only knows how to wait.

So whenever there is a RAW hazard:

👉 Processor inserts stalls until value is fully written back.

Example (Forwarding OFF)

add r1, r2, r3
sub r4, r1, r5

Now sub cannot get value early.

It must wait completely.

So several stall cycles are inserted.

This is called:

✅ Stall-only processor
✅ Simpler processor design
❌ Slower performance

6. Easy Comparison

| Feature | Forwarding ON | Forwarding OFF |
| --- | --- | --- |
| Processor behavior | Smart modern CPU | Simple old-style CPU |
| RAW hazards | Mostly solved | Cause stalls |
| add → sub dependency | 0 stall | Multiple stalls |
| lw → use dependency | 1 stall | More stalls |
| Speed | Faster | Slower |

7. Very Easy Real-Life Analogy

Imagine cooking food.

Without Forwarding

Person B must wait until Person A completely finishes and puts food on table.

👉 Waiting happens = stalls

With Forwarding

Latency = waiting time before next instruction can use the result.

Example:

add r1, r2, r3
sub r4, r1, r5

Suppose add has latency = 3 cycles.

That means:

sub cannot safely use r1 until 3 cycles later.

In Stall-Only Processor (Forwarding OFF)

Rule says:

Stalls = latency - 1

So if latency is 3:

Stalls = 3 - 1 = 2

Processor inserts 2 waiting cycles.

Why minus 1?

Because dependent instruction is already naturally one cycle behind in pipeline.

Only extra waiting cycles are counted as stalls.

Easy Example Table

| Instruction Type | Example Latency | Stall Cycles (Forwarding OFF) |
| --- | --- | --- |
| add | 2 | 1 stall |
| mul | 4 | 3 stalls |
| load (lw) | 2 | 1 stall |

With Forwarding ON

Latency mostly does not matter for normal arithmetic instructions because forwarding gives result early.

But for load instructions:

lw r1, 0(r2)
add r3, r1, r4

Data comes from memory late.

So even with forwarding:

✅ 1 stall still needed.

# Feature 2: Critical Path Analysis

1. What is Critical Path?

Critical path means:

👉 The longest dependency chain of instructions.

It is the sequence of instructions that controls the minimum possible execution time.

Processor cannot finish program faster than this chain because every next instruction depends on previous ones.

Easy Example

1. lw  r1, 0(r2)
2. add r3, r1, r4
3. mul r5, r3, r6
4. sub r7, r5, r8

Here:

- add depends on lw

- mul depends on add

- sub depends on mul

So they form one dependency chain.

This chain is the critical path.

Why is it Important?

Even if processor is super fast:

❌ It still cannot execute dependent instructions together.

Because each instruction must wait for previous result.

So critical path gives:

✅ The theoretical fastest possible execution.

2. What is “Weighted Path Length”?

Not every instruction takes same time.

Example:

| Instruction | Latency |
| --- | --- |
| add | 1 cycle |
| mul | 3 cycles |
| lw | 2 cycles |

Critical path length is calculated using these latencies.

Example:

lw  = 2
add = 1
mul = 3
sub = 1

Total path:

2 + 1 + 3 + 1 = 7 cycles

So critical path length = 7 cycles.

3. What Does the Algorithm Do?

The tool builds a dependency graph.

Each instruction is connected to instructions depending on it.

Then:

Step 1: Topological Sort

Processor arranges instructions in dependency order.

Meaning:

Instruction comes before instructions that depend on it.

Step 2: Process Backwards

Tool starts from end instructions and moves backward.

Why?

Because it wants to know:

👉 “How much work remains after this instruction?”

Step 3: Compute Best Path

For each instruction:

Path Length =
Its Latency + Largest Successor Path

Example:

lw  -> add -> mul

Latencies:

lw  = 2
add = 1
mul = 3

Now backward:

mul = 3
add = 1 + 3 = 4
lw  = 2 + 4 = 6

So longest path = 6 cycles.

Step 4: Find Largest Path

Instruction with biggest value becomes start of critical path.

Step 5: Backtrack Path

Tool follows biggest successors to reconstruct actual chain.

4. Why Red Border in GUI?

Instructions on critical path are highlighted because:

👉 They are the bottleneck.

Improving them gives biggest performance improvement.

5. What is “Critical path: 10 instructions, 18 cycles min”?

Meaning:

- Critical chain contains 10 dependent instructions

- Their total weighted latency = 18 cycles

So:

❌ Program can NEVER execute in less than 18 cycles

Even with perfect scheduling.

6. What is Theoretical Minimum CPI?

CPI means:

Cycles Per Instruction

Formula:

CPI = Total Cycles / Number of Instructions

Theoretical Minimum CPI

This means:

👉 Best possible CPI if scheduling was perfect.

Calculated using critical path.

Example:

Instructions = 22
Critical Path Cycles = 18

Then:

CPImin=1822=0.818

Meaning:

Even ideal scheduling cannot go below 0.818 CPI.

7. What is Scheduling Gap?

Scheduling gap means:

👉 How far current schedule is from perfect schedule.

Formula:

Scheduling Gap =
Actual CPI - Theoretical Minimum CPI

Example:

Suppose:

Actual CPI = 1.000
Minimum CPI = 0.818

Then:

Scheduling Gap=1.000-0.818=0.182

So tool shows:

Scheduling gap: +0.182 CPI above min

Meaning:

Processor is taking 0.182 more cycles per instruction than the best theoretically possible execution.

8. Very Easy Analogy

Imagine making tea.

Some tasks depend on others:

Boil water → add tea → pour tea

You cannot pour tea before boiling water.

That dependency chain is the critical path.

Even if 10 people help:

❌ Tea cannot finish before water boils.

So:

- Critical path = unavoidable minimum time

- Theoretical min CPI = best possible efficiency

- Scheduling gap = extra inefficiency above ideal execution

# Feature 3: Stall Profile

1. What is Stall Profile?

Previously, the tool only showed:

Total stalls = 8

But that does not explain:

- Which instructions caused stalls

- Why stalls happened

- Which hazards were removed after optimization

So Stall Profile gives detailed information about every stall.

2. What Information Does a Stall Event Store?

Each stall event stores:

| Field | Meaning |
| --- | --- |
| Producer instruction | Instruction producing data |
| Consumer instruction | Instruction using that data |
| Hazard type | Type of dependency problem |
| Stall cycles | How many cycles wasted |
| Source text | Original assembly instructions |

3. Producer and Consumer Meaning

Producer

Instruction that creates a value.

Example:

add r1, r2, r3

This produces r1.

So this is producer instruction.

Consumer

Instruction that uses that value.

sub r4, r1, r5

This uses r1.

So this is consumer instruction.

4. Example Stall Event

lw  r1, 0(r2)
add r3, r1, r4

Here:

- lw produces r1

- add consumes r1

This creates load-use hazard.

The stall profile may store:

| Field | Value |
| --- | --- |
| Producer | lw r1,0(r2) |
| Consumer | add r3,r1,r4 |
| Hazard Type | RAW Load-Use |
| Stall Cycles | 1 |
| Eliminated | No |

5. What is Hazard Type?

Hazard type tells:

👉 What kind of dependency caused the stall.

Usually here:

RAW (Read After Write)

Meaning:

Instruction tries to read value before previous instruction finishes writing it.

6. Why is Stall Profile Useful?

It helps students understand:

✅ Exactly where performance is being lost
✅ Which instructions are causing delays
✅ Whether optimization improved pipeline behavior

7. Optimization Example

Original code:

lw  r1, 0(r2)
add r3, r1, r4

This causes 1 stall.

Optimized code:

lw  r1, 0(r2)
add r7, r8, r9
add r3, r1, r4

Now independent instruction is inserted between them.

So load gets enough time to finish.

Result:

✅ Stall removed

8. Eliminated Column Meaning

The GUI compares:

- Original instruction sequence

- Optimized instruction sequence

If same producer-consumer pair no longer stalls after optimization:

Table shows:

Eliminated = YES

Meaning:

✅ Optimization successfully removed that stall.

9. Easy Real-Life Analogy

Suppose in restaurant:

- Chef cooks food → producer

- Waiter serves food → consumer

If waiter arrives too early:

❌ Must wait
→ stall happens

Manager now adds another task in between:

Cook food
Pack order
Serve food

Now waiter does not wait anymore.

That delay was eliminated.

10. Overall Simple Meaning

Stall Profile is basically:

A detailed report showing:

- which instructions caused stalls,

- why they happened,

- how many cycles were wasted,

- and whether optimization removed them.

# Feature 4: Register Pressure Visualization

1. What is Register Pressure?

Register pressure means:

👉 How many variables (registers) are “alive” at the same time during program execution.

Simple meaning:

More live values = more load on CPU registers.

If too many values are needed at once, CPU registers become full → performance problems.

2. What is a “Live Register”?

A register is live if:

- It has been written before

- It will be used again in the future

- Its value is still needed

Easy Example

1. add r1, r2, r3
2. add r4, r1, r5
3. add r6, r4, r7

Now look step by step:

- After instruction 1 → r1 is live (needed by instruction 2)

- After instruction 2 → r1 still live, r4 is live (needed by instruction 3)

- After instruction 3 → r6 becomes live

So at some points multiple registers are active together.

3. What is Register Pressure?

At any point:

Register Pressure = number of live registers

Example:

At a certain point:

- r1 is live

- r4 is live

- r6 is live

👉 Pressure = 3

4. Why is Register Pressure Important?

Because CPU has limited registers.

If pressure becomes too high:

❌ CPU cannot keep all values in registers
❌ Some values must be moved to memory

This is called:

👉 Spilling

Spilling is slow because memory access is much slower than registers.

5. Liveness Rule (Simple Explanation)

At position i:

A register is live if:

- It was written earlier

- It will be used later

So we check:

👉 “Is this value still needed in future?”

If yes → live
If no → dead

6. Register Pressure Example (Step-by-step)

1. add r1, r2, r3
2. add r2, r1, r4
3. add r3, r2, r5
4. add r4, r3, r6

Now track live registers:

After instruction 1:

- r1 is needed later
👉 Pressure = 1

After instruction 2:

- r1 (used later)

- r2 (used later)
👉 Pressure = 2

After instruction 3:

- r1 still needed indirectly

- r2 still needed

- r3 needed later
👉 Pressure = 3

After instruction 4:

- r4 live

- r3 still live
👉 Pressure = 2

So pressure rises and falls depending on dependencies.

7. GUI Color Meaning

| Pressure | Color | Meaning |
| --- | --- | --- |
| 0–4 | Green | Safe, enough registers |
| 5–8 | Orange | Moderate pressure |
| 9+ | Red | Danger of spilling |

8. Why Scheduling Affects Register Pressure?

When we optimize code:

Good side:

- Fewer stalls

- Better performance

Bad side:

- More instructions overlap in time

- More values stay alive longer

So:

👉 Scheduling trades stalls for register usage

9. Simple Analogy

Think of registers like chairs in a room.

- Each live variable needs a chair

Low pressure (green)

4 chairs, 3 people

✔ Everyone sits comfortably

Medium pressure (orange)

8 chairs, 7–8 people

⚠ Room is getting full

High pressure (red)

10 chairs, 12 people

❌ Not enough chairs → some people must stand (spill to memory)

10. Bar Visualization Meaning

The GUI shows a bar:

bar width = (pressure / 16) * 40 pixels

Meaning:

- 0 pressure → no bar

- 16 pressure → full bar

- anything above → overflow risk

Tooltip shows:

👉 “You currently have X live registers”

11. Final Simple Summary

Register Pressure Visualization tells:

How many values are alive at each instruction point, and whether the CPU is at risk of running out of registers.

It helps you understand:

- where register usage is high

- where spilling might happen

- how scheduling affects register demand

Main solution: Register Spilling

When registers are not enough:

👉 CPU/compiler moves some values to memory (RAM) temporarily.

This is called:

Spilling

Example:

If you need 12 live registers but CPU has only 8:

- 8 stay in registers

- 4 are stored in memory

- later loaded back when needed

✔ Solves overflow
❌ Slower (because memory is slow)

2. Compiler Optimization (Reduce register pressure)

Compilers try to reduce pressure before spilling.

(a) Better Instruction Scheduling

Reorder instructions so values die earlier.

Example idea:

Instead of keeping r1 alive for long time, use it early and free it.

✔ reduces number of live registers

(b) Dead Code Elimination

Remove unused calculations:

x = a + b;
x = 10;   // previous value useless

✔ reduces live registers

(c) Register Allocation Smartness

Compiler tries to:

- reuse same register for different variables

- assign registers efficiently

Example:

r1 used for A
later reused for B after A is dead

✔ reduces pressure without spilling

(d) Loop optimization (important)

Loops increase pressure because values stay alive longer.

Optimizations:

- loop unrolling carefully

- breaking dependencies

- reordering instructions

3. Hardware solution (limited)

CPU itself cannot “fix” pressure, but helps indirectly:

- more registers (modern CPUs have many)

- register renaming (in advanced CPUs)

4. Trade-off idea (VERY IMPORTANT)

Register pressure is always a balance:

| Goal | Effect |
| --- | --- |
| Reduce stalls | may increase register pressure |
| Reduce pressure | may increase stalls |

So compiler tries to find a sweet spot

5. Simple real-life analogy

Think:

- Registers = seats in a bus

- Variables = passengers

If too many passengers come:

Solution options:

- Put some passengers in another bus (spill to memory)

- Change timing so not everyone comes together (optimization)

- Reuse seats when someone leaves (register reuse)

6. Final simple answer

Yes, solutions exist:

✔ Register spilling (main solution)
✔ Compiler optimizations (reduce pressure)
✔ Better scheduling and allocation
✔ More hardware registers (partial solution)

But:

👉 You cannot fully avoid register pressure — only manage it efficiently.

# Feature 5: Loop-Aware Basic Block Splitter

1. What is a Basic Block?

A basic block is a simple group of instructions where:

👉 Execution enters at the top
👉 Executes straight line
👉 Leaves only at the end (no jump in middle)

So:

A basic block = instructions with no jumps or branches inside.

Easy Example

1. add r1, r2, r3
2. sub r4, r1, r5
3. mul r6, r4, r7

This is a basic block because:

- No jumps

- No labels

- Just straight execution

2. Why Do We Need Basic Blocks?

Without splitting, the scheduler thinks:

👉 “All instructions are one big sequence”

This is bad because:

❌ It may incorrectly reorder across jumps
❌ It breaks program logic
❌ It ignores control flow

So we divide program into smaller safe pieces.

3. What is Loop-Aware Basic Block Splitter?

It means:

👉 The program is divided into basic blocks
👉 But it also understands loops and branches
👉 So it does NOT break correctness

4. Why “Loop-Aware”?

Because loops repeat execution.

Example:

LOOP:
    add r1, r2, r3
    sub r4, r1, r5
    beq r4, r0, LOOP

This is a loop block.

Splitter must:

✔ recognize loop structure
✔ not mix instructions from different iterations
✔ keep loop logic correct

5. Rules of Basic Block Splitting

Rule 1: First instruction is a block leader

👉 Program always starts a block

Example:

Block 1 starts at first instruction

Rule 2: Labels or branch targets start new blocks

Example:

LOOP:
add r1, r2, r3

LOOP: starts a new block because execution can jump here.

Rule 3: After branch/jump → new block starts

Example:

add r1, r2, r3
beq r1, r0, EXIT
sub r4, r5, r6

After beq, execution may jump elsewhere.

So next instruction starts new block.

Rule 4: Each block gets its own scheduler

This is very important:

👉 Each block is optimized separately

So:

- Dependency graph is built per block

- Scheduling is done per block

✔ safer optimization
✔ better accuracy

Rule 5: Blocks are reassembled in original order

After optimization:

👉 Blocks are joined back in same program flow order

So correctness is preserved.

6. Simple Example (Before Split)

1. add r1, r2, r3
2. sub r4, r1, r5
3. beq r4, r0, L1
4. mul r6, r4, r7
5. L1: add r8, r9, r10

After Splitting:

Block 1

add r1, r2, r3
sub r4, r1, r5
beq r4, r0, L1

Block 2

mul r6, r4, r7

Block 3

L1: add r8, r9, r10

7. Why This Improves Scheduling?

Now scheduler can:

✔ reorder instructions safely inside blocks
✔ compute correct dependencies
✔ avoid breaking jumps/branches

But cannot:

❌ move instructions across blocks (unsafe)

8. Real-Life Analogy

Think of a program like a road trip.

- Each basic block = one city route

- Inside city → you can rearrange travel order freely

- Between cities → you cannot change path order

Without splitting:

You think whole country is one road → confusion, wrong shortcuts

With splitting:

You treat each city separately → safe and optimized travel inside each city

9. Final Simple Meaning

Loop-aware basic block splitter:

It divides the program into safe straight-line sections (basic blocks), making sure loops and jumps are respected, and then optimizes each section separately without breaking program logic.

This GUI line is basically showing how each basic block behaves before and after optimization.

Let’s break it simply:

1. “Basic blocks: 2”

👉 Your program was split into 2 separate sections (basic blocks).

So instead of one long instruction flow, the scheduler now sees:

- B1 (Block 1)

- B2 (Block 2)

Each block is optimized separately.

2. What is “Per block CPI contribution”?

CPI means:

Cycles Per Instruction (average time per instruction)

So this line shows:

👉 How much each block contributes to performance (before → after optimization)

Format:

B1 = 1.059 → 1.000
B2 = 1.000 → 1.000

3. Meaning of B1 = 1.059 → 1.000

Before optimization:

- Block 1 was slower

- CPI = 1.059

👉 This means:

On average, each instruction in B1 took 1.059 cycles (because of stalls, hazards, bad scheduling)

After optimization:

- CPI becomes 1.000

👉 Meaning:

Now block runs perfectly (ideal case)

- No wasted cycles

- No extra stalls (or minimized to best possible)

4. Meaning of B2 = 1.000 → 1.000

👉 Block 2 was already optimal

So:

- Before: perfect (1.000)

- After: still perfect (1.000)

✔ No improvement needed
✔ No hazards or stalls to fix

5. Overall Meaning (Simple)

Your program has:

Block 1

- Had inefficiencies

- Scheduler improved it

- Became optimal

Block 2

- Already optimal

- No change

6. Very Easy Analogy

Think of 2 rooms cleaning work:

Room 1 (B1)

- Messy before cleaning

- After cleaning → perfectly organized

Room 2 (B2)

- Already clean

- No improvement needed

7. What does this tell you in exam terms?

This output shows:

✔ Program is divided into independent execution regions
✔ Optimization improved only the problematic region
✔ Scheduler successfully reduced CPI to ideal value (1.0) in B1
✔ B2 had no performance bottleneck

8. Final Simple Interpretation

The program has 2 basic blocks. Block 1 was optimized from CPI 1.059 to perfect 1.0 (removing stalls/inefficiencies), while Block 2 was already optimal so it stayed unchanged.